Project Overview

This project extends the StarCore-1 baseline RISC processor developed in Practical 4 into the FPC-16 Fixed-Point Precision Core. The StarCore-1 is a 16-bit single-cycle Harvard-architecture processor implemented in Verilog HDL. While functional for integer arithmetic, it lacks native support for decimal calculations — a critical requirement for the OrbitEdge-1 CubeSat mission, which requires sensor data processing, attitude determination, and telemetry checksum computation involving fractional values.

The FPC-16 extension addresses this by introducing native 16-bit fixed-point arithmetic using the Q8.8 format, where the upper 8 bits represent the integer part and the lower 8 bits represent the fractional part. This allows decimal values to be represented and computed entirely in hardware without software emulation, reducing both execution time and power consumption — essential for the 400 mW power budget of the OrbitEdge-1 mission.

Two new instructions are introduced using the currently undefined opcodes 1110 and 1111: FPMUL (fixed-point multiply) and FPDIV (fixed-point divide). Fixed-point addition and subtraction reuse the existing ADD and SUB instructions since binary addition and subtraction operate correctly on Q8.8 encoded values without modification.

Progress Summary

3 Completed Tasks

1. Project Scoping and Fixed-Point Format Selection

The group has selected the Q8.8 fixed-point format as the encoding standard for the FPC-16. This decision was made after analysing the constraints of the StarCore-1's 16-bit datapath. Q8.8 provides 8 bits for the integer part (range 0–255) and 8 bits for the fractional part (precision of 1/256 ≈ 0.004), which fits precisely within the existing 16-bit registers without requiring any changes to the register file or memory organisation. This format is sufficient for the target application domain of satellite sensor data processing.

Furthermore, it was determined that fixed-point addition and subtraction are mathematically equivalent to integer addition and subtraction in binary — the existing ADD and SUB instructions therefore require no modification and can be used directly for fixed-point addition and subtraction. Only multiplication and division require new hardware due to the need for intermediate bit-width expansion and post-operation shifting.

2. ISA Extension Design

The StarCore-1 ISA was analysed to identify available opcode space. The 4-bit opcode field supports 16 possible values (0000–1111). Of these, 13 are defined instructions, opcode 1010 is reserved for co-processor integration, and opcodes 1110 and 1111 are currently undefined. These two opcodes have been assigned to the new fixed-point instructions:

| Opcode | Mnemonic | Operation |
| --- | --- | --- |
| 1110 | FPMUL | Q8.8 fixed-point multiply |
| 1111 | FPDIV | Q8.8 fixed-point divide |

Both instructions follow the existing R-type encoding format: [15:12] opcode | [11:9] RS1 | [8:6] RS2 | [5:3] WS | [2:0] unused. This ensures minimal changes to the instruction decoder and maintains backward compatibility with all existing StarCore-1 programs.

3. Initial FPMUL Module Implementation

An initial implementation of the FixedPointMul.v module has been completed. The Q8.8 multiplication algorithm works as follows:

- Two 16-bit Q8.8 inputs a and b are multiplied to produce a 32-bit intermediate result

- The correct Q8.8 result occupies bits [23:8] of the 32-bit product (shift right by 8)

- Bits [31:24] represent overflow and bits [7:0] represent sub-precision fractional bits that are discarded

The initial implementation is shown below:

verilog

// =========================================================================

// FixedPointMul.v — Q8.8 Fixed-Point Multiplier

// FPC-16 Extension for StarCore-1

// =========================================================================

// Multiplies two Q8.8 fixed-point numbers.

// Q8.8 format: bits[15:8] = integer part, bits[7:0] = fractional part

// Full product is 32 bits; result is extracted from bits[23:8]

// =========================================================================

module FixedPointMul (

input  [15:0] a,        // Q8.8 fixed-point operand A

input  [15:0] b,        // Q8.8 fixed-point operand B

output [15:0] result    // Q8.8 fixed-point result

);

wire [31:0] full_product;

assign full_product = a * b;

assign result = full_product[23:8];  // shift right by 8 bits

endmodule

Initial simulation confirms correct behaviour for representative test cases:

- 2.0 × 3.0 = 6.0 ✅ (0x0200 × 0x0300 = 0x0600)

- 1.5 × 2.0 = 3.0 ✅ (0x0180 × 0x0200 = 0x0300)

- 1.5 × 1.5 = 2.25 ✅ (0x0180 × 0x0180 = 0x0240)

3 Planned Tasks

1. Implement and Verify FixedPointDiv Module

The next immediate priority is implementing FixedPointDiv.v. Q8.8 division requires pre-shifting the dividend left by 8 bits before performing integer division to maintain fractional precision:

result = (a << 8) / b

This will be implemented as a combinational module and verified with a dedicated testbench covering standard cases, division by 1, and near-zero divisors.

2. Integrate FPMUL and FPDIV into StarCore-1 Datapath

The following modules will be updated to support the two new instructions:

- ALU.v — add cases for 3'b??? FPMUL and FPDIV operations

- ALU_Control.v — add casex entries for opcodes 1110 and 1111

- ControlUnit.v — add case entries asserting R-type control signals for new opcodes

Since FPMUL and FPDIV follow R-type encoding, the control signals are identical to existing R-type instructions (RegDst=1, ALUSrc=0, RegWrite=1, ALUOp=00), minimising the changes required.

3. Write Integration Test Program and Run Full Simulation

A new test.prog will be written that exercises all four fixed-point operations:

- Load two Q8.8 values from memory

- Perform FPADD (using ADD), FPSUB (using SUB), FPMUL, FPDIV

- Store results back to memory

- Verify correct values using the integration testbench

This will demonstrate end-to-end correctness of the FPC-16 extension.

Design Documentation

Block Diagram

The FPC-16 extends the StarCore-1 datapath with two new functional units inside the ALU. The overall processor hierarchy remains unchanged:

StarCore1 (top level)

├── ControlUnit (CU) — extended for opcodes 1110, 1111

└── Datapath (DU)

├── InstructionMemory — unchanged

├── GPR — unchanged

├── ALU_Control — extended for opcodes 1110, 1111

├── ALU — extended with FPMUL and FPDIV units

│   ├── FixedPointMul — NEW

│   └── FixedPointDiv — NEW (planned)

└── DataMemory — unchanged

[Block diagram figure to be inserted — showing new FPMUL/FPDIV units inside ALU]

Architectural Schematic

The Q8.8 multiplication schematic:

a [15:0] ──┐

├──► 16×16 multiplier ──► full_product [31:0]

b [15:0] ──┘                              │

bits [23:8]

│

result [15:0]

The 32-bit intermediate product is produced by multiplying the two 16-bit inputs. Bits [23:8] of the result correspond to the correctly scaled Q8.8 output.

Behavioral Flowchart — FPMUL Instruction Execution

START

↓

Fetch instruction (opcode=1110)

↓

ControlUnit decodes: R-type signals

(RegDst=1, ALUSrc=0, RegWrite=1)

↓

GPR reads RS1 and RS2 (Q8.8 values)

↓

ALU_Control decodes opcode 1110 → FPMUL

↓

FixedPointMul computes full_product = RS1 × RS2

↓

Extract result = full_product[23:8]

↓

MemToReg=0 → write ALU result to WS register

↓

PC = PC + 2

↓

END

Evidence of Work

Code Snippet 1 — FixedPointMul.v

verilog

module FixedPointMul (

input  [15:0] a,

input  [15:0] b,

output [15:0] result

);

wire [31:0] full_product;

assign full_product = a * b;

assign result = full_product[23:8];

endmodule

Code Snippet 2 — ALU_Control.v extension (planned)

verilog

6'b001110: ALU_Cnt = 3'b??? ; // FPMUL

6'b001111: ALU_Cnt = 3'b??? ; // FPDIV

Initial Simulation Results

Simulation of FixedPointMul.v confirms correct Q8.8 multiplication:

| Test | a (Q8.8) | b (Q8.8) | Expected | Observed | ✅ |
| --- | --- | --- | --- | --- | --- |
| T1 | 0x0200 (2.0) | 0x0300 (3.0) | 0x0600 (6.0) | 0x0600 | ✅ |
| T2 | 0x0180 (1.5) | 0x0200 (2.0) | 0x0300 (3.0) | 0x0300 | ✅ |
| T3 | 0x0180 (1.5) | 0x0180 (1.5) | 0x0240 (2.25) | 0x0240 | ✅ |

Conclusion

The FPC-16 project is progressing on schedule. The fixed-point format has been selected, the ISA extensions have been defined, and the initial FPMUL module has been implemented and verified. The immediate next steps are implementing FPDIV, integrating both modules into the StarCore-1 datapath, and running a full integration test. The modular design of the StarCore-1 baseline significantly reduces integration complexity, as only the ALU, ALU_Control, and ControlUnit require modification.